ID=

rho = fluid density
Qr = rated flow rate
Hr = rated pump head
eta_pump = pump efficiency

Ps_r = rated suction pressure
Pd_r = rated discharge pressure

Nr = rated pump speed

Tf_r = rated fluid temperature

Q = actual flow rate
Ps = suction pressure
Pd = discharge pressure
N = pump RPM

Vrms = RMS vibration
Vpeak = peak vibration
Vstruct = structural vibration

T_bearing = bearing temperature
T_motor = motor temperature
T_casing = pump casing temperature

T_oil = oil temperature
T_fluid = fluid temperature
T_ambient = ambient temperature

P_oil = oil pressure
Level_oil = oil level

P_seal = seal pressure
Leakage = leakage rate

P_motor_r = rated motor power
V_r = rated voltage

PF = motor power factor
eta_motor = motor efficiency

I_actual = motor current

Nmax = maximum allowed pump speed
SF = motor service factor

Vrms_max = max allowable RMS vibration
Vpeak_max = max allowable peak vibration

T_bearing_max = max bearing temperature
T_motor_max = max motor temperature
T_casing_max = max casing temperature

T_oil_max = max oil temperature
T_fluid_max = max fluid temperature

P_oil_min = minimum oil pressure
Level_min = minimum oil level

P_seal_max = max seal pressure

Leakage_max = max allowable leakage

Runtime_since_maintenance
Runtime_maintenance_interval

Days_since_maintenance
Days_maintenance_interval

StartStop_actual
StartStop_allowed

Pmax = maximum pressure in time window
Pmin = minimum pressure in time window

Interruptions_count = number of flow interruptions

Acoustic_actual = measured acoustic signal level

# Minimum Core Set (Most Important)

If you want the smallest dataset that still works, these are the critical variables:

rho
Qr
Hr
Nr

Q
Ps
Pd
N

P_motor_r
V_r

T_bearing
T_motor
T_oil

Vrms

Runtime_since_maintenance
StartStop_actual

These alone can already estimate pump load, stress, and health indicators.

If you want, I can also show you the correct way to structure these variables into a dataset for your predictive maintenance model (which is slightly different from the raw SCADA layout). That step is usually where most systems get it wrong.

d

- ID

- pressure

- flow_rate

- suction_pressure

- discharge_pressure

- rpm

- vibration_rms

- vibration_peak

- structural_vibration

- motor_current

- power_draw_kw

- bearing_temperature

- motor_temperature

- pump_casing_temperature

- oil_temperature

- fluid_temperature

- ambient_temperature

- oil_pressure

- oil_level

- seal_pressure

- leakage_rate

- runtime_hours_since_last_maintenance

- days_since_last_maintenance

- start_stop_count

- pressure_fluctuations

- flow_interruptions

- acoustic_signal_level

## pressure

P = rho * g * H

rho = fluid density
g = gravitational acceleration
H = pump head

## flow_rate

Q_expected = Qr * (Na / Nr)

Qr = rated flow rate
Na = actual pump speed
Nr = rated pump speed

## suction_pressure

L_suction = Ps_min / Ps_actual

Ps_min = minimum safe suction pressure
Ps_actual = measured suction pressure

## discharge_pressure

Pd_max = Ps_max + rho * g * Hr * (Nmax / Nr)^2

Ps_max = maximum suction pressure
rho = fluid density
g = gravitational acceleration
Hr = rated pump head
Nmax = maximum pump speed
Nr = rated pump speed

## rpm

L_rpm = N_actual / N_max

N_actual = measured pump speed
N_max = maximum allowed pump speed

## vibration_rms

L_vibration_rms = V_rms_actual / V_rms_max

V_rms_actual = measured RMS vibration
V_rms_max = allowable RMS vibration

## vibration_peak

L_vibration_peak = V_peak_actual / V_peak_max

V_peak_actual = measured peak vibration
V_peak_max = allowable peak vibration

## structural_vibration

L_structural_vibration = V_struct_actual / V_struct_max

V_struct_actual = measured structural vibration
V_struct_max = allowable structural vibration

## motor_current

I_r = (P_motor_r * 1000) / (sqrt(3) * V_r * PF * eta_motor)

P_motor_r = rated motor power
V_r = rated voltage
PF = motor power factor
eta_motor = motor efficiency

## power_draw_kw

P_draw = (rho * g * Q * H) / (eta_pump * eta_motor)

rho = fluid density
g = gravitational acceleration
Q = flow rate
H = pump head
eta_pump = pump efficiency
eta_motor = motor efficiency

## bearing_temperature

L_bearing_temp = (T_bearing_actual - T_ambient) / (T_bearing_max - T_ambient)

T_bearing_actual = measured bearing temperature
T_bearing_max = maximum allowable bearing temperature
T_ambient = ambient temperature

## motor_temperature

L_motor_temp = (T_motor_actual - T_ambient) / (T_motor_max - T_ambient)

T_motor_actual = measured motor temperature
T_motor_max = maximum motor temperature
T_ambient = ambient temperature

## pump_casing_temperature

L_casing_temp = (T_casing_actual - T_ambient) / (T_casing_max - T_ambient)

T_casing_actual = measured casing temperature
T_casing_max = maximum allowable casing temperature
T_ambient = ambient temperature

## oil_temperature

L_oil_temp = (T_oil_actual - T_ambient) / (T_oil_max - T_ambient)

T_oil_actual = measured oil temperature
T_oil_max = maximum allowable oil temperature
T_ambient = ambient temperature

## fluid_temperature

L_fluid_temp = T_fluid_actual / T_fluid_max

T_fluid_actual = measured fluid temperature
T_fluid_max = maximum allowable fluid temperature

## ambient_temperature

T_motor_max_effective = T_motor_max - Ka * (T_ambient - T_ambient_rated)

T_motor_max = rated maximum motor temperature
T_ambient = measured ambient temperature
T_ambient_rated = design ambient temperature
Ka = temperature derating constant

## oil_pressure

L_oil_pressure = P_oil_min / P_oil_actual

P_oil_min = minimum safe oil pressure
P_oil_actual = measured oil pressure

## oil_level

L_oil_level = 1 - (Level_actual - Level_min) / (Level_full - Level_min)

Level_actual = measured oil level
Level_min = minimum safe oil level
Level_full = full oil level

## seal_pressure

P_seal_min = P_process + deltaP

P_process = process pressure
deltaP = required pressure margin

## leakage_rate

L_leakage = Leakage_actual / Leakage_max

Leakage_actual = measured leakage rate
Leakage_max = allowable leakage rate

## runtime_hours_since_last_maintenance

L_runtime = Runtime_since_maintenance / Runtime_maintenance_interval

Runtime_since_maintenance = operating hours since last maintenance
Runtime_maintenance_interval = scheduled maintenance interval

## days_since_last_maintenance

L_days = Days_since_maintenance / Days_maintenance_interval

Days_since_maintenance = days since last maintenance
Days_maintenance_interval = scheduled maintenance interval

## start_stop_count

L_start_stop = StartStop_actual / StartStop_allowed

StartStop_actual = number of start stop cycles
StartStop_allowed = allowed start stop cycles

## pressure_fluctuations

L_pressure_fluct = (P_max - P_min) / DeltaP_allowed

P_max = maximum pressure in time window
P_min = minimum pressure in time window
DeltaP_allowed = allowable pressure variation

## flow_interruptions

L_flow_interruptions = Interruptions_count / Interruptions_allowed

Interruptions_count = number of flow interruptions
Interruptions_allowed = allowable interruption count

## acoustic_signal_level

L_acoustic = Acoustic_actual / Acoustic_max

Acoustic_actual = measured acoustic signal level
Acoustic_max = allowable acoustic level